1. According to the Chicago Convention, if a Contracting State cannot fully comply with an ICAO Standard, it shall:

A. Apply Recommended Practices instead

B. Notify ICAO according to Article 38 of the Convention

C. Request approval from IATA

2. Which ICAO Annex primarily contains provisions regarding aircraft accident and incident investigations?

A. Annex 12

B. Annex 13

C. Annex 19

3. ICAO Document 4444 mainly addresses:

A. Air Traffic Management procedures

B. Aircraft type designators

C. Safety Management Systems

4. Under ISA conditions established by ICAO, sea level pressure is:

A. 1000 hPa

B. 1013.25 hPa

C. 1025 hPa

5. Which organization has legal competence to issue Acceptable Means of Compliance (AMC) and Guidance Material (GM)?

A. ICAO Council

B. European Commission

C. EASA Executive Director

6. Under Regulation (EU) 2018/1139, AMC and GM:

A. Create additional mandatory requirements

B. Provide a presumption of compliance with the applicable regulations

C. Replace Implementing Rules

7. Certification Specifications (CS) introduced through Regulation (EU) No. 748/2012 are intended to:

A. Replace Part-21 requirements

B. Ensure uniform application of common rules

C. Define maintenance privileges

8. Which Part contains requirements for Combined Airworthiness Organisations?

A. Part-CAMO

B. Part-CAO

C. Part-T

9. The Airworthiness Review Certificate for aircraft managed under continuing airworthiness requirements is primarily associated with:

A. Part-M and Part-CAMO

B. Part-147 and Part-66

C. Part-21 and Part-145

10. Which Regulation establishes requirements for flight crew licensing?

A. Regulation (EU) No. 965/2012

B. Regulation (EU) No. 1178/2011

C. Regulation (EU) No. 1321/2014

11. Under Regulation (EU) No. 376/2014, occurrence reporting is intended primarily to:

A. Allocate legal responsibility

B. Support disciplinary investigations

C. Prevent accidents and incidents

12. Which ICAO Annex addresses Security against acts of unlawful interference?

A. Annex 16

B. Annex 17

C. Annex 18

13. A Design Organisation Approval (DOA) is granted under:

A. Part-145

B. Part-21

C. Part-66

14. Which organization issues Certification Authorizations (CA) to certifying staff?

A. EASA

B. Competent Authority

C. Approved maintenance organization

15. Practical experience required for obtaining an AML under Part-66 is normally gained in organizations approved under:

A. Part-145 or Part-CAO

B. Part-147 only

C. Part-M exclusively

16. Which ICAO document contains location indicators for aerodromes?

A. Doc 8643

B. Doc 8585

C. Doc 7910

17. The requirements for maintenance organization approvals are contained in:

A. Part-145

B. Part-66

C. Part-147

18. Which Annex of Regulation (EU) No. 1321/2014 concerns certifying staff?

A. Annex II

B. Annex III

C. Annex IV

19. A Maintenance Training Organisation approved under Part-147 may contract practical training activities to:

A. ICAO-approved organizations

B. Part-145 or Part-CAO organizations

C. Air operators only

20. Which document provides detailed radiotelephony procedures?

A. ICAO Doc 9432

B. ICAO Doc 9859

C. ICAO Doc 8973

21. According to Part-66, the Aircraft Maintenance Licence itself automatically grants certification privileges:

A. In all circumstances

B. Only after company authorization and compliance with applicable requirements

C. Immediately after examination success

22. Which Part contains requirements for third-country aircraft continuing airworthiness?

A. Part-T

B. Part-ML

C. Part-M

23. Which Regulation governs Air Operations requirements?

A. Regulation (EU) No. 1178/2011

B. Regulation (EU) No. 965/2012

C. Regulation (EU) No. 376/2014

24. Under Part-66, type training is normally provided by an organization approved in accordance with:

A. Part-21

B. Part-M

C. Part-147

25. Which ICAO Annex covers Environmental Protection?

A. Annex 15

B. Annex 16

C. Annex 18

26. According to ISA, the standard density at MSL is approximately:

A. 1.125 kg/m³

B. 1.225 kg/m³

C. 1.325 kg/m³

27. Which statement concerning AMC is correct?

A. Compliance with AMC is mandatory by law.

B. Alternative means of compliance may be used if accepted.

C. AMC are issued by the European Commission.

28. The relationship between Part-CAMO and Part-145 is best described as:

A. Part-145 issues work orders to CAMO.

B. CAMO manages continuing airworthiness and tasks maintenance organizations.

C. Both have identical responsibilities.

29. Which Part establishes requirements for continuing airworthiness of non-complex aircraft not listed in an AOC?

A. Part-ML

B. Part-M

C. Part-T

30. Under Part-66, a certifying staff member exercising certification privileges must hold:

A. An ICAO personnel licence only

B. An Aircraft Maintenance Licence and appropriate company authorization where required

C. A Part-21 approval

31. Under Part-66, possession of an Aircraft Maintenance Licence (AML) alone:

A. Automatically permits CRS issuance in any Part-145 organisation

B. Is a necessary but insufficient condition for CRS privileges

C. Replaces the need for Certification Authorization

32. The Certification Authorization (CA) is issued by:

A. The competent authority

B. EASA Headquarters

C. The approved maintenance organisation

33. The Certificate of Release to Service (CRS) is legally considered a document of:

A. The certifying staff

B. The maintenance organisation

C. The competent authority

34. Category B1 automatically includes:

A. Category B2 privileges

B. The corresponding Category A subcategory

C. Category C privileges

35. Which category does NOT automatically include any Category A privileges?

A. B1

B. B2

C. C

36. In a Part-145 organisation, Category A privileges are limited to:

A. Base maintenance only

B. Scheduled maintenance and avionics troubleshooting

C. Minor scheduled line maintenance and rectification of simple defects

37. A Category B1 certifying staff member may certify replacement of avionics LRUs provided:

A. Complex troubleshooting is required

B. Functionality can be checked by simple tests

C. Software loading is involved

38. Category B2 privileges primarily relate to:

A. Structures and powerplants

B. Mechanical and hydraulic systems only

C. Avionics and electrical systems

39. Category C certifying staff issue CRS following:

A. Line maintenance only

B. Scheduled base maintenance supported by B1/B2 support staff

C. Any maintenance without support

40. Support staff under Part-145 are generally holders of:

A. Category A licences only

B. Category B1 and/or B2 licences

C. Category L licences

41. Basic examinations and training used to support an AML application must have been completed within:

A. Five years before application

B. Eight years before application

C. Ten years before application

42. An AML becomes invalid:

A. Five years after its last issue or change unless presented to the issuing authority for verification

B. Five years after first issue irrespective of amendments

C. Three years after issue unless renewed

43. Candidates proven to have cheated during a Part-66 examination shall be prohibited from taking further examinations for:

A. Six months

B. Twelve months

C. Twenty-four months

44. According to 66.B.500, which action by a licence holder constitutes grounds for suspension, limitation or revocation of the AML?

A. Refusing overtime work

B. Issuing CRS while adversely affected by alcohol or drugs

C. Working without type training on non-complex aircraft under supervision