Q1. Which Subpart establishes procedures for issuing Certificates of Airworthiness?
A) Subpart H
B) Subpart I
C) Subpart J

Q2. A repair to an ETSO article (other than an APU) is treated as:
A) A standard repair
B) A maintenance action
C) A change to the ETSO design

Q3. Which Subpart establishes Design Organisation Approval (DOA)?
A) Subpart F
B) Subpart J
C) Subpart O

Q4. What is the purpose of CS-STAN?
A) Standards for large aeroplanes
B) Standards for standard changes and standard repairs that do not require approval when conditions are met
C) Standards for noise certification only

Q5. Which of the following aircraft categories can qualify for a standard change under 21.A.90B?
A) Rotorcraft with an MTOM of 3,000 kg
B) Aeroplanes with an MTOM of 10,000 kg
C) Large transport aeroplanes only

Q6. What is the scope of Part 21 Subpart A (21.A.1)?
A) Aircraft maintenance procedures
B) General rights and obligations of certificate applicants and holders
C) Pilot licensing requirements

Q7. Subpart D (21.A.90A) deals with:
A) Noise certificates
B) Changes to type-certificates and restricted type-certificates
C) Permits to fly

Q8. Aircraft identification plates must be:
A) Painted on the fuselage
B) Fireproof and permanently secured
C) Removed during maintenance

Q9. Under Subpart F, the manufacturer must demonstrate conformity of:
A) Products, parts, and appliances with applicable design data
B) Pilot training programs
C) Airport operating procedures

Q10. Which certification specification applies to turbine-powered large aeroplanes?
A) CS-23
B) CS-25
C) CS-27

Q11. Which Subpart establishes procedures for ETSO authorisations?
A) Subpart O
B) Subpart Q
C) Subpart B

Q12. A Permit to Fly may be issued for:
A) Flying an aircraft to a maintenance location
B) Scheduled commercial passenger service only
C) Pilot licensing examinations only

Q13. According to 21.A.3A, occurrence reports are collected and analyzed primarily to:
A) Increase aircraft production rates
B) Identify adverse trends and address deficiencies
C) Reduce certification costs

Q14. What is the primary purpose of a Production Organisation Approval (POA)?
A) Approve flight crews
B) Show conformity of products, parts, and appliances with design data
C) Issue Certificates of Airworthiness

Q15. Which combination correctly matches the certification specification and aircraft type?
A) CS-25 — Large turbine-powered aeroplanes
B) CS-27 — Sailplanes
C) CS-22 — Large rotorcraft

Q16. Standard changes under 21.A.90B may apply to aeroplanes with an MTOM of:
A) 5,700 kg or less
B) 8,618 kg or less
C) 3,175 kg or less

Q17. Which information must be included in product identification under 21.A.801?
A) Manufacturer’s name, product designation, and serial number
B) Pilot’s name and license number
C) Aircraft registration only

Q18. Which Subpart establishes Production Organisation Approval (POA)?
A) Subpart G
B) Subpart H
C) Subpart M

Q19. Which Subpart establishes procedures for issuing type-certificates and restricted type-certificates?
A) Subpart B
B) Subpart F
C) Subpart K

Q20. CS-27 certification specifications apply to rotorcraft with:
A) More than nine passenger seats
B) Maximum weight of 3,175 kg or less and nine or fewer passenger seats
C) Maximum weight of 8,618 kg

Q21. A standard change cannot be applied if it:
A) Uses approved design data
B) Conflicts with the TC holder's data
C) Includes instructions for continued airworthiness

Q22. A repair is defined as:
A) Any maintenance action
B) Elimination of damage and/or restoration to an airworthy condition after release to service
C) Replacement of fuel and oil

Q23. The 'EXEMPT' mark may appear on an engine identification plate when:
A) The manufacturer requests it
B) The competent authority grants an exemption from environmental protection requirements
C) The aircraft is privately owned

Q24. Noise certificates are covered under:
A) Subpart K
B) Subpart I
C) Subpart P

Q25. A Supplemental Type-Certificate (STC) is primarily used for:
A) Approval of major changes to a type-certificate
B) Issuing a Certificate of Airworthiness
C) Approving standard repairs only

Q26. CS-22 applies to sailplanes and powered sailplanes with a maximum of:
A) Two occupants
B) Four occupants
C) Six occupants

Q27. For a change to be considered a standard change, it must:
A) Be approved by the TC holder
B) Follow design data included in Agency certification specifications
C) Always require an STC

Q28. What does Subpart E establish?
A) Production approvals
B) Supplemental Type-Certificate (STC) procedures for major changes
C) Airworthiness review procedures

Q29. Under 21.A.2, who may undertake actions required of a certificate holder?
A) Any person without restrictions
B) Only EASA inspectors
C) Another natural or legal person under an agreement ensuring obligations are met

Q30. CS-23 applies to normal-category aeroplanes with:
A) Up to 19 passenger seats and a maximum certified take-off mass of 8,618 kg or less
B) Up to 50 passenger seats
C) Unlimited passenger seats