- Three capacitors, each having a capacitance of 12 µF, are connected in series. What will be the total capacitance?
a) 12 µF
b) 4 µF
c) 36 µF

- If both the plate area and the dielectric thickness of a capacitor are doubled, the capacitance becomes:
a) Twice the original value
b) Unchanged
c) Half the original value

- An electrolytic capacitor is mainly required in applications where:
a) Very low losses are needed
b) Current flow must follow a fixed polarity
c) Alternating current is used

- Two capacitors of 15 µF each are connected in parallel. What is the equivalent capacitance?
a) 15 µF
b) 7.5 µF
c) 30 µF

- A ceramic capacitor with a high dielectric constant (ε) is generally preferred:
a) In applications requiring minimum losses
b) In high-frequency electronic circuits
c) In polarized current applications

- A polyester capacitor has the following color bands from top to bottom: BLACK – RED – ORANGE – GREEN – BROWN. Using the standard color code values (black = 0, brown = 1, red = 2, orange = 3, green = 5), the capacitor specification is:
a) 3,000 nF with 5% tolerance and 100 V rating
b) 2,000 pF with 5% tolerance and 100 V rating
c) 12,000 mF with 5% tolerance and 100 V rating

- In a circuit where two capacitors are connected in parallel, the equivalent capacitance is:
a) Less than the smaller capacitor value
b) Greater than the larger capacitor value
c) Dependent on the capacitor values

- The most common fault in capacitors is usually caused by:
a) Increase in plate thickness
b) Loss of dielectric insulation
c) Reduction in supply voltage

- Deterioration of the dielectric in a capacitor may occur because of:
a) Increased conductor size
b) Humidity and temperature conditions
c) Magnetic shielding

- Capacitors that are most likely to suffer dielectric failure are:
a) Ceramic capacitors only
b) Air capacitors only
c) Paper and electrolytic capacitors

- Before testing a capacitor with an instrument, it should always be:
a) Heated slightly
b) Connected to AC supply
c) Fully discharged

- A swollen electrolytic capacitor usually indicates that it should be:
a) Recharged
b) Replaced immediately
c) Connected in series

- A capacitance meter is mainly used to measure the:
a) Current rating
b) Resistance of wires
c) Actual capacitance value

- In low-capacitance worn capacitors, the capacitance generally tends to:
a) Remain constant
b) Decrease
c) Increase

- A multimeter used in ohmmeter mode can help determine whether a capacitor is:
a) Producing magnetic flux
b) Functioning properly
c) Increasing voltage

- During capacitor testing with an ohmmeter, the resistance initially becomes low because the capacitor is:
a) Discharging permanently
b) Charging
c) Open circuited

- If the resistance never returns to maximum during testing, the capacitor is likely:
a) Fully insulated
b) Perfectly normal
c) Short-circuited

- If the resistance remains constantly at maximum during testing, the capacitor is:
a) Fully charged
b) Short circuited
c) Open circuited

- During the charging process of a capacitor, the charging current is initially:
a) Constant at minimum value
b) Maximum
c) Zero

- A capacitor stores energy in the form of:
a) Mechanical energy
b) Magnetic energy
c) Electrostatic energy

- In a DC circuit, capacitors are commonly used to:
a) Generate magnetic fields
b) Store electrical energy
c) Increase mechanical power

- The capacitance of a capacitor increases when the plate area is:
a) Kept unchanged
b) Increased
c) Reduced

- If the distance between capacitor plates increases, the capacitance will:
a) Increase
b) Remain the same
c) Decrease

- A dielectric material with a higher dielectric constant produces a capacitor with:
a) Lower capacitance
b) Zero capacitance
c) Greater capacitance

- The dielectric strength of an insulating material indicates:
a) Its physical weight
b) The resistance of the connecting wires
c) The maximum electric field it can withstand

- How should bar magnets be properly kept when not in use?

a) Stored separately on wooden racks

b) Placed in pairs with unlike poles facing each other and joined by keepers

c) Hung vertically using strings

- Which notation represents the permeability of a material in absolute terms?

a) μ

b) μr

c) μ0

- At which location is the magnetic influence of Earth considered the greatest?

a) Near the equatorial region

b) Around the magnetic poles

c) In the middle latitudes

- Which material is commonly selected for making electromagnet cores because it magnetizes quickly?

a) Hardened steel

b) Soft iron

c) Brass

- What can be said about the retentivity property of iron?

a) It is very weak

b) It is absent completely

c) It is extremely strong

- In a magnetic field outside a magnet, the magnetic lines of force travel:

a) From the north pole toward the south pole

b) Randomly in all directions

c) Across each other repeatedly

- Which of the following substances can be magnetized with the greatest ease?

a) Iron

b) Copper

c) Plastic

- In an alternating-current generator, permanent magnets mainly help in:

a) Producing the magnetic field required for generation

b) Changing AC into DC

c) Regulating voltage automatically

- What is the primary cause of eddy currents in a conductor?
a) Constant electric resistance
b) A changing magnetic field
c) Mechanical vibration

- Why are laminated metal cores used in electrical machines?
a) To reduce eddy current losses
b) To increase conductor thickness
c) To improve insulation only