Human Factors Accident Summary Table

| Year/Date | Airline & Aircraft | Route (From → To) | Main Cause of Accident |
| --- | --- | --- | --- |
| August 1985 | Japan Airlines Boeing 747-100 | Tokyo → Osaka | Incorrect repair of rear pressure bulkhead years earlier after a tail scrape. Fatigue crack developed, causing decompression, loss of stabilizer and hydraulics, leading to crash. |
| 1983 | Eastern Airlines Lockheed L-1011 | Miami → Nassau (returning to Miami) | Magnetic Chip Detectors (MCDs) installed without O-rings, causing oil leakage from all three engines. Also involved organizational and procedural failures. |
| April 1988 | Aloha Airlines Boeing 737-200 | Hilo → Honolulu | Failure to detect fatigue damage and disbonding in fuselage lap joints during inspections, causing explosive decompression. |
| 10 June 1990 | British Airways BAC 1-11 (Flight 5390) | Birmingham → (emergency landing at Southampton) | Windscreen installed with incorrect bolts (84 of 90 undersized). Human factors included poor lighting, fatigue, perceptual errors, and lack of independent checks. |
| 8 January 2003 | Air Midwest Beech 1900D (Flight 5481) | Charlotte, North Carolina → Scheduled destination | Incorrect rigging of elevator control system during maintenance and aircraft overload with aft center of gravity. Maintenance procedure was misunderstood and incomplete. |

Super Short Exam Revision

| Aircraft | Key Human Factor |
| --- | --- |
| Japan Airlines B747 (1985) | Incorrect repair |
| Eastern Airlines L-1011 (1983) | Missing O-rings during maintenance |
| Aloha Airlines B737 (1988) | Inspection failure |
| British Airways BAC 1-11 (1990) | Wrong bolts fitted |
| Air Midwest Beech 1900D (2003) | Incorrect control rigging |

Easy Memory Trick

R-O-I-B-R

R = Repair error → Japan Airlines

O = O-ring missing → Eastern Airlines

I = Inspection failure → Aloha Airlines

B = Bolt error → British Airways

R = Rigging error → Air Midwest

.